You are my AI engineering partner.
We are developing a next-generation AI-assisted electronic circuit design application that combines:

Core technologies

- SPICE simulation (ngspice or Xyce via Python)

- Symbolic analysis using SymPy / Lcapy (MNA-based)

- AI reasoning + LLM agent

- Automated circuit optimization

- GUI schematic editor (PySide6 / Qt)

- Vendor model integration (TI, ADI, LTspice libraries)

Primary goal of the product

Build an intelligent circuit simulator that can:

- Automatically design circuits (e.g., audio amplifiers)

- Tune parameters such as gain, noise, THD, bandwidth, power consumption

- Generate and modify SPICE netlists

- Run SPICE simulations

- Interpret results

- Automatically adjust components or even change the topology

- Iterate until the design meets the user’s specs

AI assistant capabilities

You must act as a technical co-designer with the ability to:

- Read high-level user design requests (“reduce noise by 3 dB”, “improve THD”, “increase gain”)

- Generate symbolic transfer functions and small-signal models

- Perform sensitivity analysis

- Modify component values based on symbolic reasoning and simulation feedback

- Optimize multi-objective performance

- Explain design decisions clearly

- Suggest alternative topologies when needed

- Integrate vendor SPICE models automatically

- Guide implementation in Python, Qt, SymPy, SPICE, data structures, and workflows

GUI & UX goals

Work with me to design:

- A modern schematic editor (PySide6/Qt)

- Parameter control panels

- Optimization workflow UI

- Design history / AI reasoning logs

- Model manager (vendor libraries, custom models)

Future extension

We plan to integrate this as a Cadence add-on, extending AI-assisted design to ASIC analog circuitry using SPICE-level simulation and symbolic approximations.

Your role

From now on, always stay fully within this project context and help me with:

- Product design

- MVP definition

- Architecture diagrams and system design

- UI/UX layout ideas

- Optimization strategies

- Technical implementation details (Python + Qt + SPICE + SymPy)

- Algorithms for iterative tuning, convergence, stability

- Competitive analysis and differentiation

- Pricing and business model

- Marketing content for landing pages, demos, and pitch decks

Important

Every answer must assume we are actively building this AI circuit designer together.
Do not drift outside the project.
Do not provide generic answers.
All outputs must be practical, detailed, and implementation-oriented.

Begin by confirming understanding and then continue helping me build this product.

If you'd like, I can also prepare:
✅ a short version
✅ a high-temperature creativity version
✅ a developer-only technical spec version
Just let me know!

Below is a clear, realistic, low-risk MVP plan for our AI-assisted circuit design application.
This version is optimized for fastest path to a working product, minimal engineering burden, and maximum early user value.

🚀 MVP PLAN — AI-Assisted Circuit Designer

🎯 MVP Goal

Deliver a functional application where a user can:

- Draw or load a simple circuit (op-amp, filter, transistor amplifier)

- Tell the AI what they want (“increase gain to 40 dB”, “reduce noise”, “improve THD”)

- The system runs SPICE, analyzes results, adjusts components, and iterates.

- The result is shown visually and numerically.

This is enough to demo the vision and onboard early adopters.

1. MVP CORE FEATURES

✅ 1. SPICE Simulation Pipeline (Foundation)

- Use ngspice (Python bindings via PySpice or subprocess + raw netlists).

- Support:

  - AC analysis

  - DC operating point

  - Transient (basic)

- Parser for SPICE outputs:

  - Gain at a frequency

  - Noise summary

  - THD extraction via FFT (Python/Numpy)

Deliverable:
simulate(netlist) → metrics dict {gain, bandwidth, noise, THD}

✅ 2. Netlist Generator

Start with text-based circuit definition:

- Simple components (R, C, L, op-amp macro, MOSFET)

- Node naming

- Python object → SPICE string

Later replaced by a full schematic editor.

Deliverable:
circuit.to_spice()

✅ 3. Symbolic Engine (Minimal Version)

Provide symbolic transfer function only for:

- RC filters

- Op-amp stages

- Single-transistor amplifiers

Use:

- SymPy or Lcapy

Purpose:

- Extract symbolic gain

- Compute ∂gain/∂R, ∂gain/∂C, etc.

- Suggest which component to adjust

Deliverable:
symbolic_analysis(circuit) → sensitivities

✅ 4. AI Reasoning Agent (Core Logic)

The agent performs:

- Understand user intent: “increase gain to 40 dB”

- Call symbolic engine → find sensitive components

- Modify components

- Run SPICE

- Compare results vs target

- Iterate

Algorithm:
Hybrid symbolic + numerical optimization loop

Deliverable:
optimize(circuit, user_goals) → updated circuit

✅ 5. GUI (Minimal) — PySide6

Instead of a full schematic editor, MVP GUI includes:

- Component list (editable table)

- Netlist view

- Simulation result plots (Plotly)

- AI command box (“reduce THD”, “increase bandwidth”)

- Optimization progress window

Full graphical nodes/wires can come in v1.1 or v2.

Deliverable:
MVP GUI with AI command interface + plots + live updates

✅ 6. Pre-built Circuit Templates

To showcase value and reduce friction:

- Non-inverting op-amp amplifier

- Sallen-Key filter

- Common-source MOS amplifier

- LM358 & TL072 based audio preamp

- Audio output stage (simple class-AB)

Deliverable:
Button: “Start from Template → Optimize”

2. END-TO-END FLOW OF MVP

Step 1: User selects a circuit template

Step 2: Circuit loaded into table-based editor

Step 3: User types:

👉 “Increase gain to 40 dB”

Step 4: AI agent converts intent → target spec

Step 5: Symbolic engine selects parameter to adjust

Step 6: SPICE simulation → performance measured

Step 7: AI adjusts component → re-runs SPICE

Step 8: When spec met → show optimized design

This simple loop is enough to demonstrate breakthrough functionality.

3. WHAT WE EXCLUDE FROM MVP

These are deferred to later versions:

❌ Full GUI schematic editor
❌ Multi-objective optimization
❌ AI topology generation from scratch
❌ Vendor model auto-download
❌ Integration into Cadence
❌ Large component libraries
❌ PCB integration
❌ Machine learning model of circuits
❌ AI chat with memory
❌ Collaboration features

These slow down development and aren’t needed for early validation.

4. TECHNICAL ARCHITECTURE (MVP VERSION)

┌───────────────────────────────────────────┐

│                 GUI (Qt)                  │

│  - component table                        │

│  - netlist viewer                         │

│  - AI command box                         │

│  - plot panel (Plotly)                    │

└───────────────────────────────────────────┘

│

▼

┌───────────────────────────────────────────┐

│            AI Reasoning Agent             │

│ - interprets user goals                   │

│ - calls symbolic engine                   │

│ - makes component adjustments             │

│ - drives optimization loop                │

└───────────────────────────────────────────┘

│

┌───────────┴───────────┐

▼                       ▼

┌──────────────────┐   ┌──────────────────┐

│ Symbolic Engine   │   │  SPICE Engine    │

│ (SymPy/Lcapy)     │   │ (ngspice/Xyce)   │

└──────────────────┘   └──────────────────┘

│                       │

└───────────┬──────────┘

▼

┌────────────────────────┐

│ Component Update Logic  │

└────────────────────────┘

5. MVP RELEASE SCOPE & TIMELINE

Week 1–2

- SPICE backend working

- Circuit objects + netlist generator

- Basic GUI with component table

Week 3–4

- Symbolic engine prototype

- AI agent → adjust gain only

- Basic plots

Week 5–6

- Support optimization of THD, bandwidth, and noise (simple versions)

- Add templates

- End-to-end loop

- Polish GUI

Week 7

- Testing, stability, docs

- Launch MVP demo

Total: ~6–7 weeks (realistic for a two-person team).

6. WHY THIS MVP WILL SUCCEED

Because it already delivers the unique value:
⚡ AI-assisted circuit tuning
⚡ Automatic SPICE iteration
⚡ Symbolic reasoning for smart component changes
⚡ A usable GUI for non-programmers

This is far beyond LTspice / TINA / PSpice and no competitor offers anything similar.

If you want, I can now create:

🔹 Detailed task breakdown (Jira-ready)
🔹 Gantt chart
🔹 MVP architecture diagram (UML)
🔹 Wireframes for GUI
🔹 Feature-by-feature implementation guide

Just tell me.

Context for SPICE model support in my app (PLEASE READ CAREFULLY)

I’m building an AI-assisted circuit design app that uses SPICE in the loop (currently ngspice) to evaluate and optimize circuits (gain, bandwidth, noise, THD, etc.).

While testing, I discovered an important limitation:

1. Problem: Vendor SPICE models are not all ngspice-compatible

Many vendor op-amp models from Texas Instruments / Analog Devices / LTspice are not plain SPICE3. They often use:

- PSpice-only “A devices” (e.g. A... SCHMITT, A... OTA, TABLE sources, etc.)

- Special behavioral constructs (Dnlim(), Uplim(), %vd, %id, etc.)

- LTspice-specific primitives and syntax

- Sometimes encrypted or compiled model blocks

These models:

- Work in PSpice, LTspice, vendor tools

- But fail in ngspice with errors like “unable to find definition of model ...” or “unknown device Axxx”.

So ngspice cannot directly run many TI / ADI / LTspice models, especially modern, complex macromodels like TL072.301 or OP284.lib.

For my product, it is crucial that users can import TI / ADI / LTspice vendor models, so I cannot rely on “ngspice only”.

2. High-level solution: Multi-simulator + model-compatibility layer

I want to design the app so it can robustly handle vendor models by using a combination of:

- Multiple simulator backends

  - ngspice → fast, open-source, good for standard SPICE models

  - Xyce (or similar) → supports many PSpice/LTspice constructs and advanced behavioral models better than ngspice

- Model compatibility detection

  - When a user imports a .lib / .cir / .sub model, the app should:

    - Scan the file for “problematic” elements (A-devices, special functions, encrypted sections, etc.)

    - Classify the model as:

      - “Standard SPICE” → safe for ngspice

      - “PSpice-like” → likely better in Xyce

      - “LTspice-specific” → needs conversion or Xyce fallback

      - “Unsupported/encrypted” → cannot be simulated (warn the user)

- Automatic simulator routing

  - If model is standard SPICE → run on ngspice

  - If model uses PSpice/LTspice extensions but is still mostly analog → try Xyce

  - If neither can handle it as-is → attempt model simplification/conversion (see next point)

- Model simplification / conversion layer (optional but powerful)

  - Implement a translator that can:

    - Detect unsupported A-devices / behavioral constructs

    - Replace them with equivalent or approximate circuits using only standard SPICE elements (E/G/B sources, RC networks, etc.)

  - The goal is not 100% exact behavior, but “good enough” for:

    - Small-signal AC gain / bandwidth

    - Time-domain behavior for THD/noise analysis

  - Once converted, cache the simplified model for future use.

- Fallback + user feedback

  - If a model truly cannot be supported (e.g. encrypted, heavily digital, mixed-signal), the app should:

    - Clearly inform the user which parts are unsupported

    - Offer to fall back to a generic op-amp / transistor macromodel

    - Still allow symbolic + AI-guided design, just not with that exact vendor model

3. What I want from you (the AI assistant) in this chat

Help me:

- Design the “Model Compatibility Engine”:

  - Rules/heuristics for detecting:

    - standard SPICE models

    - PSpice/LTspice-only features

  - Data structures to store model metadata (vendor, type, compatibility flags, preferred simulator, etc.)

- Design the multi-simulator architecture:

  - How to abstract the simulator interface so I can plug in ngspice and Xyce behind a common API

  - How to route a given simulation job (netlist + models) to the appropriate backend based on model analysis

- Define the user workflow for importing models:

  - How users select a vendor .lib / .cir / .sub file

  - How the app analyzes it and reports compatibility

  - How the app chooses ngspice vs Xyce vs simplified model

  - How errors / warnings are communicated

- Plan future extensions:

  - Support for encrypted / proprietary models where possible

  - Handling of mixed-signal or digital-heavy models

  - Integration with a local model library (TI, ADI, LTspice, etc.)

Important:
All of this should be designed with my existing app in mind:

- Python backend

- Circuit object → netlist generator

- SPICE-in-the-loop optimizer (gain, bandwidth, noise, THD, etc.)

- Planned GUI for circuit editing and model management

Please:

- Assume this is part of a serious commercial-grade tool

- Focus on practical, implementable solutions

- Be explicit about limitations of ngspice / Xyce vs PSpice/LTspice

- Propose clear class designs, APIs, and workflow diagrams where useful.